Installation of GP32 PC-Link Host application

To install GP32 PC-Link Host application, just execute the install program. (iGpW2kEn.exe for Windows 2000 and iGpW98En.exe for Windows 98) Then you may see following display.

And you may be asked to select the location where the program will be installed.

Select the location as you want and click “Next” button. Then the installation process will begin. It will take just a few seconds to be completed.

Installation of GP32 USB Driver

You should install the USB driver for GP32 machine to use GP32 PC-Link Host application. To do this, follow next steps.

Windows 98

- Turn on your GP32 machine and connect the USB cable with it.

- Then, Windows 98 will find GP32 machine and show following message box.

- Click “Next” button.

- Select “Search for the best driver for your device. (Recommended).” and click “Next” button.

- Among 4 check boxes, select only “Specify a location” and click “Browse…” button. Then you may see a dialog box as follows.

- Now you should select the location where the proper drivers are. As default, these drivers are stored in the folder “Drivers” under the location where you installed GP32 PC-Link Host application. After the folder highlighted, click “OK” button.

- Now you can check the driver file is selected. Click “Next” button. Then Windows 98 will show following message.

- Click “Next” button. The Windows 98 may install this driver.

- Click “Finish” button.

Windows 2000

- Turn on your GP32 machine and connect the USB cable with it. Then Windows 2000 will show following message.

- Click “Next” button.

- Select “Search for a suitable driver for my device (recommended) option and click “Next” button.

- Select only “Specify a location” option and click “Next” button.

- Click “Browse” button.

- Now you should specify the location where the driver files are. GP32 USB driver files are stored in the folder “Drivers” under the location where you installed GP32 PC-Link Host application.

- Select that location and click “Open” button.

- Click “OK” button.

- Now you’re ready to installl the driver. Click “Next” button then Windows will start to install the driver.

10. Installation completed. Just click “Finish” button.

Executing & Using

If you execute GP32 PC-Link Host application, it will show a message as follows.

Then, you should connect the USB cable to GP32 machine and set its mode to PC-Link mode. If ready, press OK button.

If you execute the application for the first time (in other words, if you didn’t create any signature file yet), it will ask you if you want to create a signature file now. You don’t have to create one right now, but it will ask you everytime you execute the application until you create one.

Signature file When you perchase a GP32 machine, you should register your machine ID in server. This information will be used to encrypt GP32-specific application when you download any application from server. If you don’t register this information, you cannot download any GP32-specific application from server. To do this, you should first create a signature file and register your machine ID in GP32 website using this file.

If you select “Yes”, it will show a dialog box.

You can set the ID and password to be stored in GP32 machine now. But you can change these informations at any time by using main menu. Refer to the “menus” section to do this.

ID & password stored in GP32 machine To connect to Internet using mobile phone, you should first connect to RAS(Remote Access Service) server. theses ID & password are used by the server to authenticate the user.

If you complete entering ID and password and press “OK” button, it will ask you where you want to store this file.

By defauit, the folder where the signature file will be stored is set as the folder “gps” located under the folder where you installed GP32 PC-Link Host program (in this case, it is “D:\Program Files\Gamepark\GP32 PC-Link Host\gps”). But you can store this signature file in any folder you want. But be aware not to forget the location where you store this file.

If you complete to create the signature file, the ID and password you just entered will be shown as follows.

Now, you can see the main window.

You can handle the files and folders in SMC as with Windows Explorer. You can copy, paste or delete the files or folders in SMC. But there are some restrictions.

Restrictions in using GP32 PC-Link Host application - You CANNOT copy the files or folders from SMC to SMC or from PC to PC. - You CANNOT delete the files or folders in PC. You should do this using Windows Explorer. - There are some restrictions in the format of files and folders in SMC. First, the file name MUST be equal or less than 8 characters long. Second, the extension of the file name in GP32 machine MUST be equal or less than 3 characters long. - There are 5 default folders in SMC. We recommend you that you don’t remove these folders if the SMC is used for GP32 machine. They are “GAME”, “MP3”, “GPSYS”, “GPMM” and “GPETC”. The usage of these folders will be described soon. (If you format the SMC using this application, it will automatically create these folders for you.) - There are some characters which cannot be used for file name.

GP32-specific default folders - GAME : All the GP32-specific applications (e.g. games, text viewer, etc.). Be careful not to remove any files under this folder. It may cause the applications not to execute properly. - MP3 : All the mp3 music files should be stored in this folder. If not, the MP3 player cannot find them. We recommend that you copy the mp3 files using “MP3 Basket” menu. For more details, refer to “MP3 Basket” section in this manual. - GPSYS : Where GP32-specific system files are stored. Be careful not to remove the files in this folder. It may cause some application not to execute properly. - GPMM : For future support. Some multimedia files will be stored here. - GPETC : For future support. Currently, this folder is used for TextViewer. For details, refer to the TextViewer manual.

Example : Copying a file from PC to SMC.

- Select a source file which will be copied to SMC. This file MUST obey the restrictions mentioned above.

- Copy this file. There are four ways for copying.

- using shortcut (Ctrl-C)

- using Toolbar

- using main menu

- using context-sensitive menu (which will pop up if you click right mouse button in file list panel.)

- Select a target folder where the copied file will be pasted.

- Paste the copied file to the target folder. There are also four ways to do so.

- Then, the application will start to copy this file showing the progress as follows.

6. If completed, the application will show the updated file list as follows.

Menus

- Main menu

- File menu

  - New folder : Creates new folder in SMC.

  - Delete : Deletes file(s) or folder(s) in SMC. You cannot delete file(s) or folder(s) on PC.

  - Format : Formats SMC in GP32-specific format. (This means it creates 5 default folders as well as formats SMC)

  - Install : Installs a GP32-specific application (e.g. game) in SMC. You need a GP32 package file

GP32 package file Package file for GP32 machine. It has *.zpk extension.

  - Uninstall : Uninstalls a GP32-specific application from SMC. If you uninstall an application, you CANNOT reinstall it unless you have the package file.

  - Back Up : Backs up a GP32-specific application from SMC to PC. You can reinstall the application just by copying the files back to SMC in any time.

  - Properties : Shows the SMC space information in status bar.

  - Close : Disconnect from GP32 machine and terminates the application.

- Edit menu

  - Copy : Copies selected file(s) or folder(s) in SMC or PC.

  - Paste : Pastes copied file(s) or folder(s) into target folder.

Warning ! You CANNOT copy file(s) or folder(s) from SMC to SMC or PC to PC. You can just copy from SMC to PC or PC to SMC.

- View menu

  - Toolbars : Shows or hides the toolbars.

  - Status Bar : Shows or hides the status bar.

  - Large Icons : Shows the file list with large icons.

  - Small Icons : Shows the file list with small icons.

  - List : Shows the file list in list format.

  - Details : Shows the file list with detailed informations (Name, Type, Size and Modified date)

  - Arrange Icons : Sorts the file list by designated criteria. (by Name, Type, Size or Modified date)

  - Refresh : Shows the file list with most recent information.

- Configuration menu

  - Mode : Sets the mode. There are two modes. Details follows.

GP32-specific modes - GP mode : Default mode. In GP mode, you cannot see the files or folders which don’t need to be seen for you. We recommend you to use the application with this mode. - normal mode : You can see all the files and folders in SMC. In other words, you can use the application like a SMC reader/writer.

- GP32 specific

  - MP3 basket : Used for sending MP3 files to SMC. Refer to “MP3 basket” section for details.

  - ID & password : Changes ID & password stored in GP32 machine.

  - Signature file : Makes signature file and sets ID & password.

- Help menu

  - About GP32 PC-Link Host : Shows application information.

- Context-sensitive menu

- This menu will pop up when you right-click your mouse in file list. It has several submenus as follows. The functions of these submenus are already explained above.

- New Folder

- View

- Arrange Icons

- Refresh

- Paste

- Copy

- Delete

Toolbar

Toolbar has seven buttons. Their functions follow.

: Moves up to parent folder.

: Copies file(s) or folder(s) in SMC or PC.

: Pastes file(s) or folder(s) in SMC or PC.

: Deletes selected file(s) or folder(s) in SMC.

: Refresh the file list currently shown.

: Sets the way file list is shown. (Large Icons/ Small Icons/ List/ Details)

: Creates new folder in SMC.

MP3 Basket

- Why do you need this facility?

There are two restrictions for the mp3 files to be played properly by GP32 machine. They are “file name restriction” and “mp3 tag restriction”. You may know about the “file name restriction” by the explanation above. So I will explain only the “mp3 tag restriction”.

There are two mp3 tag standards currently. They are Id3v1 and Id3v2. (For more technical details, refer to the site “http://www.id3v2.org”) You can see these information on some mp3 players working on PC. (e.g. WinAmp™). But MP3 player embedded in GP32 machine cannot recognize the Id3v2 tag. So you should remove this tag before you copy the mp3 file to SMC. This is a somewhat bothering work. So we provide this tool to make it easy to copy mp3 files. If you use this supplementary tool, you can handle the mp3 files more easily.

<tag information shown by WinAmp>

Main Window

If you select “GP Specific  MP3 Basket”, the main window will pop up as follows.

In the left file list, all folders and mp3 files in default mp3 folder on PC are shown. You can set the default mp3 folder on PC by pressing the button “set this folder as default” when current folder is what you want to set as default(You can check current folder in the path information beyond he file list.). You can add the mp3 files to the waiting list (right file list) by double-clicking the file or selecting file(s) and pressing “>” button.

If you add mp3 files to the list, there may appear icons on the left side of each file. These icons indicate the status of the mp3 file.

- : Means the mp3 file ready to be copied to SMC.

- : Means the mp3 file doesn’t have any Id3v1 tag information. You should append Id3v1 information.

- : Means the mp3 file has a too long file name. You should modify the file name length equal or less than 8 characters.

- : Worst case. Means the mp3 file doesn’t have any Id3v1 tag information AND has a too long file name.

Editing mp3 file

If you select one mp3 file in the list, you can edit the Id3v1 information or file name.

- File name : Enter the file name with which the mp3 file may be stored in SMC. File name should obey the restriction rule mentioned above. You can copy first 8 characters from the original file name by using “copy PC file name” button.

- Title : Enter the title of this mp3 file (up to 30 characters). This information will be used by GP32 MP3 player when displaying the mp3 file list in MP3 folder in SMC.

- Artist : Enter the artist of this mp3 file (up to 30 characters). This information will be used by GP32 MP3 player when displaying the mp3 file list in MP3 folder in SMC.

Title and Artist In GP32 MP3 Player, the mp3 files are displayed in the format of “Artist-Title”. If mp3 file doesn’t have any Id3v1 tag information, it will display the file name instead.

Album : Enter album information (up to 30 characters).

Year : Enter year informaition (up to 4 characters).

Genre : Select one of the pre-defined genre information.

Comment : Enter any additional information as you wish (up to 30 characters).

Additional buttons used in editing Reset : Resets the information to the original state. Back up : It is convenient to back up the modified files for future usage. You can do so by using this button. If you press this button, it will ask you the target file name with which the backup mp3 file will be stored in PC. Default target folder is set to the “mp3” folder under the location where you installed GP32 PC-Link Host application. But you can change it to any folder you want. If stored once, the mp3 file can be copied to SMC without modification. Copy ver.2 info : Not implemented yet. Copies Idv2 tag information to Idv1 tag.

If you complete editing, press “Apply” button to update the information. If not, it will not be applied. Then, you can see the icon changes.

Edit remaining files in similar way.

Now, you’re ready to send mp3 file to SMC. If all the files are in proper status, you can send them to SMC by pressing “Send” button. But be aware that free spaces are available on SMC. You can check space availability in “SMC space” information.